Specification Document — ESP32 Multi-Sensor, GPS & GSM IoT Device

# Project Overview

This device is a low-power IoT tracking and monitoring unit designed to:

- Read multiple environmental and system sensors

- Track GPS position

- Send scheduled location updates via GSM

- Respond to SMS location requests

- Sync stored history via Wi-Fi whenever available

- Be fully configurable through a built-in ESP web interface (BLE support planned later)

The system is optimized for long-term operation on battery or solar power.

# Core Functional Requirements

### Microcontroller

- ESP32 used as the main MCU.

- Must support deep sleep, Wi-Fi, BLE (for future app), and multiple GPIO.

- Stores configuration in NVS/flash.

### Sensor Monitoring

The ESP32 must support:

- Analog inputs (temperature, voltage, current sensors)

- Digital inputs (door magnetic sensor, status signals)

- I²C sensors (BME280, INA219, etc.)

- Optional external ADC if more analog channels required

Data captured must include:

- Timestamp

- Sensor reading

- Sensor type

- Input status changes (interrupt-based events)

- History must be stored locally until upload.

### GPS Tracking

- External GPS module (e.g., NEO-6M / M8N or SIM808 internal GPS).

- Must support configurable “GPS fix interval”.

- Device must store:

  - Latitude, Longitude

  - Accuracy / number of satellites

  - Timestamp

- GPS should be power-gated to reduce consumption.

### GSM Communications

- GSM module (SIM800 / SIM808 / compatible LTE IoT module).

- Required features:

  - Send scheduled location updates 4 times per day

  - Respond to incoming SMS requesting location

  - Send sensor status alerts (optional)

  - Periodically check for Wi-Fi; if found:

    - Upload stored history

    - Clear uploaded records

- GSM must be fully power-gated except during:

  - Scheduled updates

  - SMS requests

  - Wi-Fi not available & manual send required

### Wi-Fi Sync

- ESP searches for Wi-Fi periodically or on wake.

- When connected:

  - Upload location + sensor history to remote API

  - Retry if server unreachable

  - Use secure protocol (HTTPS or MQTT with TLS)

### On-Device Configuration (Web UI)

Built-in ESP web interface must allow configuring:

- Wi-Fi SSID + Password

- GSM APN, username, password

- Phone numbers for SMS commands

- Update schedule (4× daily time slots)

- GPS settings (interval, cold/hot start delay)

- Power mode (aggressive sleep / standard)

- Sensor calibration and thresholds

- Manual test “Send Now”

- Viewing stored history

- Reset to factory defaults

Website should be accessible:

- In AP mode (first boot or factory reset)

- In STA mode (normal mode)

### Power Management Requirements

The system must:

- Use ESP32 deep sleep as default state

- Use MOSFET power gating to cut power to:

  - GSM module

  - GPS module

- Support battery or solar-powered operation

- Maintain a consumption target of:

  - <150 µA in idle deep sleep

  - <10 mA average daily (depending on GSM schedule)

Additional power features:

- Scheduled GSM communication windows

- Batch transmission of data

- GPS ON only during fix cycle

- Wake sources:

  - RTC timer

  - External interrupt (sensor)

  - GSM incoming SMS (if hardware allows)

### Data Logging Requirements

Device must maintain:

- Rolling history (configurable size: 50–5000 records)

- Each record contains:

  - Timestamp

  - GPS position

  - Sensor readings

  - Event type

- Oldest records overwritten automatically when max history reached.

History must sync during Wi-Fi availability.

# Hardware Specification

### Main Components

| Component | Purpose |
| --- | --- |
| ESP32 | Main MCU, Wi-Fi, BLE, deep sleep |
| SIM800/SIM808 | GSM & SMS communication |
| GPS Module | High-accuracy location tracking |
| Battery (Li-ion / LiPo) | Power source |
| Solar charger (optional) | Recharging battery |
| Buck regulator | 5V → 3.3V conversion |
| MOSFET power switches | Control GSM/GPS power |
| Sensors | Inputs for temperature, door, voltage, etc. |
| SD card (optional) | Extended history storage |

### Electrical Interfaces

- GSM uses UART

- GPS uses UART or I²C

- Sensors use:

  - ADC (analog)

  - GPIO interrupts (digital)

  - I²C (environmental/power sensors)

# Software Architecture

### System States

- Deep Sleep (default)

- Wake for Scheduled Update

- Wake on Interrupt (sensor event)

- Wake on SMS (if enabled)

- Configuration Mode (web server active)

- Wi-Fi Sync Mode

- Error/Retry Mode

### Data Flow Summary

Sensors → ESP32 → Storage → GSM/Wi-Fi upload → Server

### GSM Upload Format (Example JSON)

{

"device": "DEVICE_ID",

"timestamp": 1732128300,

"gps": { "lat": -29.1234, "lng": 31.1234, "accuracy": 3.5 },

"sensors": {

"temp": 24.3,

"door": "closed",

"battery": 3.87

}

}

### Scheduling Requirements

- 4 configurable transmission times per day

- RTC wake-up must trigger, all these must be triggered via SMS

  - GPS fix

  - Data capture

  - GSM power-up

  - Transmission

  - GSM shutdown

  - Return to deep sleep

# Security Requirements

- HTTPS/TLS if using Wi-Fi for upload

- Config lock with admin password

- SMS commands must come from authorized numbers

- Optional device PIN to unlock configuration mode

# Mechanical Requirements

- Compact PCB

- Antenna ports for GSM & GPS

- Waterproof enclosure option

- Battery and solar connector terminals

# Acceptance Criteria

The device is complete when it can:

- Take sensor readings

- Acquire GPS fix

- Log data locally

- Send scheduled updates via GSM

- Respond to SMS requests

- Sync history over Wi-Fi

- Save & load settings through web UI

- Operate low-power for extended periods